SmartGrid AI Audit Framework

Cyber-Physical Monitoring and Audit Intelligence

COMPLETE PROJECT KNOWLEDGE DOCUMENT (v2)

A-Z Reference: Architecture, Algorithms, Datasets, Results, and Implementation

Aditya Sanjay Jadhav  (242050010)

Guide: Prof. Shrinivas Khedkar

Dept. of Computer Engineering & IT  |  VJTI Mumbai  |  M.Tech 2025-26

# 1. Project Overview

The SmartGrid AI Audit Framework is an end-to-end cyber-physical security monitoring and audit scheduling system designed for modern power grids. It combines multi-layer anomaly detection, hybrid audit scheduling, live SCADA integration, and per-feature explainability into a single unified pipeline.

## 1.1 What Problem Does It Solve?

Modern power grids are no longer isolated physical systems. They use SCADA systems, internet-connected sensors, and two-way communication to operate efficiently. This connectivity creates an attack surface that existing tools cannot adequately defend:

- Single-layer detection methods cannot distinguish physical noise from deliberate attacks

- LSTM models tuned for low false alarms miss up to 77% of stealthy attacks when used alone

- No prior system couples real-time detection to dynamic audit scheduling

- No prior system runs against live SCADA supervisory telemetry end-to-end

- No prior system provides per-feature explainability for every anomaly decision

- Fixed audit schedules waste budget on safe agents and under-audit high-risk ones

## 1.2 Research Objectives

Six formal objectives from Section 1.4 of the report:

- Objective 1: Build a multi-layer anomaly detection pipeline combining statistical deviation, dual-branch LSTM, and rule-based attack-specific classifiers

- Objective 2: Pre-train separate LSTM branches on public domain-specific datasets — UCI Electrical Grid Stability (physical branch) and UNSW-NB15 (cyber branch)

- Objective 3: Design a hybrid audit scheduler combining Q-learning (discrete escalation) with gradient descent (continuous budget allocation)

- Objective 4: Integrate the framework with live Rapid SCADA v6 for real-time 100-agent monitoring

- Objective 5: Provide per-feature XAI explainability for every anomaly flag

- Objective 6: Achieve detection accuracy above 99% with FPR below 1% on a 100-agent 24-hour evaluation

## 1.3 What Was Built

- A four-layer anomaly detection pipeline (statistical + temporal LSTM + ensemble + attack-specific sub-detectors)

- A hybrid audit scheduler (tabular Q-learning with 4D state space + gradient descent cost optimisation)

- A feature-level explainability module (top-k contributing features per alert, per domain)

- Live integration with Rapid SCADA v6 via a PowerShell bridge (670 calculated channels)

- An eight-view Next.js operator dashboard for real-time monitoring

- Complete evaluation: 100 agents, 24h simulation, 28,800 steps, 3 attack types, 5 seeds

## 1.4 Base Paper

Reference: Priyadarsini S. (2025). AI-Driven Audit Framework for Distributed Multi-Agent Smart Grids. ACM Trans. Cyber-Physical Systems.

The base paper uses single-layer deviation scoring with Q-learning-only scheduling. It achieves 98.4% accuracy, 3.2% FPR, 87.9% risk mitigation, 42.5% cost efficiency, and 93.8% audit coverage. It uses no LSTM, no SCADA integration, no explainability, and no benchmark datasets.

## 1.5 Literature Survey — Five Themes

- Theme 1 — FDI Attacks: Liu et al. (2011) theoretical model; Liang et al. (2017) topology-aware FDI; Musleh et al. (2019) survey. Gap: all treat FDI in isolation, no cyber-layer integration.

- Theme 2 — Network Intrusion: Moustafa & Slay (2015) UNSW-NB15 creation; Hink et al. (2014) smart grid IDS features. Gap: cyber features not mapped to SCADA physical context.

- Theme 3 — SCADA Anomaly Detection: Nader et al. (2014) one-class SVM; Umer et al. (2022) LSTM for SCADA time series. Gap: no dual-domain physical+cyber combined scoring.

- Theme 4 — RL for Audit Scheduling: Huang et al. (2020) Q-learning for IoT resource allocation; Li et al. (2021) RL audit scheduling in cloud. Gap: no hybrid RL+gradient, no 4D contextual state.

- Theme 5 — Explainability in Security AI: Lundberg & Lee (2017) SHAP values; Ribeiro et al. (2016) LIME. Gap: no per-feature XAI for smart grid anomaly detection.

# 2. System Architecture

The framework operates across two workspaces sharing a common FastAPI backend:

- Experiment Running Workspace: simulation engine, controlled attack injection, benchmarking

- Rapid SCADA Live Workspace: connects to Rapid SCADA v6 with 670 calculated channels

## 2.1 Four-Tier Data Flow

- Tier 1 — Data Ingestion: Rapid SCADA (100 agents, 670 channels) or simulation engine → PowerShell bridge → FastAPI backend

- Tier 2 — Feature Processing: Raw SCADA values → per-type baseline normalisation → physical deviation vector (dx) and cyber deviation vector (dy)

- Tier 3 — Detection and Audit Intelligence: Multi-layer anomaly detection + hybrid audit scheduler + XAI explainability

- Tier 4 — Operator Interface: Next.js dashboard, 8 live views across two workspaces

## 2.2 Agent Types and SCADA Channel Allocation

| Class | IDs | Count | Physical Ch | Cyber Ch | SCADA Physical Range | SCADA Cyber Range |
| --- | --- | --- | --- | --- | --- | --- |
| GEN | GEN-01 to GEN-20 | 20 | 5 | 4 | 101-160 | 501-580 |
| SUB | SUB-21 to SUB-50 | 30 | 5 | 3 | 201-290 | 601-690 |
| PMU | PMU-51 to PMU-75 | 25 | 5 | 4 | 301-375 | 701-800 |
| BRK | BRK-76 to BRK-100 | 25 | 5 | 4 | 401-475 | 801-900 |

## 2.3 Technology Stack

| Component | Technology | Role |
| --- | --- | --- |
| Backend API | FastAPI (Python/ASGI) | REST endpoints: /detect, /schedule, /explain, /agents |
| Frontend | Next.js (React) | 8-view operator dashboard, two workspaces |
| ML Framework | PyTorch (CPU-only) | LSTM training, inference, focal loss, temperature calibration |
| Database | SQLite (audit_chain.db) | Immutable append-only audit trail with SHA-256 hash chaining |
| SCADA Platform | Rapid SCADA v6 | 670 calculated channels at 127.0.0.1:10109 |
| SCADA Bridge | PowerShell | Web API polling every 5 seconds, JSON batch output |

# 3. Baselines and Normalisation

Each of the 4 agent types (GEN, SUB, PMU, BRK) has a separate hand-engineered baseline profile. Baselines define what "normal" looks like for each feature type. Every detection computation is relative to these baselines.

## 3.1 GEN Baseline Profile (Table 3.1)

| Feature | Baseline (b_j) | Threshold (tau_j) | Physical/Cyber |
| --- | --- | --- | --- |
| Voltage | 230.0 V | 10.0 V | Physical |
| Frequency | 50.0 Hz | 1.0 Hz | Physical |
| Current | 100.0 A | 20.0 A | Physical |
| Power | 23.0 kW | 5.0 kW | Physical |
| Response Time | 3.3 ms | 5.0 ms | Physical |
| Latency | 0.17 | 0.50 | Cyber |
| Packet Loss | 0.004 | 0.05 | Cyber |
| Integrity | 0.99 | 0.20 | Cyber |
| Comm Frequency | 0.79 | 0.50 | Cyber |

## 3.2 Normalisation Formula

At every timestep, each raw observed value x_j is converted to a dimensionless deviation score:

z_j  =  (x_j - b_j) / tau_j

where x_j = observed value, b_j = baseline, tau_j = threshold. This maps any feature to a common scale regardless of units (volts, amps, ms, fractions).

## 3.3 Baseline Update Rule (EMA)

Baselines are updated using an Exponential Moving Average with alpha = 0.10:

b_j(t+1) = 0.90 x b_j(t) + 0.10 x x_j(t)   if anomaly_flag = 0

b_j(t+1) = b_j(t)                             if anomaly_flag = 1

Baselines are FROZEN during detected anomalies. This prevents attack-contaminated values from corrupting the baseline and making future attacks invisible.

## 3.4 EWMA Drift Compensation

Before deviation scoring, a slow drift correction removes natural baseline drift (load shifts over hours). Rolling mean tracked with alpha_ewma = 0.30 over a 24-timestep window. 30% of the difference between rolling mean and static baseline is subtracted from the observed value. Prevents false alarms during legitimate long-term load shifts.

# 4. Threat Model and Anomaly Generation

The adversary has partial insider knowledge: knows agent identifiers and approximate operating baselines but does not know detection algorithm internals or thresholds. Three attack types are modelled, each with precise injection parameters.

## 4.1 Anomaly Generation — Experiment Mode (grid_env.py)

In the Experiment Running workspace, attacks are injected by the simulation engine (grid_env.py). Three attack models with precisely defined injection parameters:

### False Data Injection (FDI)

x_attack(t) = x(t) + 2.0 + 0.05 x t

Constant bias = 2.0 added immediately on attack start. Linear drift = 0.05 per timestep — cumulative bias grows each step. Applied to all physical features of the targeted agent. CUSUM alarm threshold (h=4.0) is reached after approximately 8 steps of sustained drift. This is the deliberate calibration — CUSUM and FDI are co-designed.

### Denial of Service (DoS)

- latency += 5.0 (immediate 5-unit latency spike)

- pkt_loss += 0.20 (capped at 1.0 — never exceeds 100%)

- integrity -= 0.50 (floored at 0 — never goes negative)

- comm_freq x= 0.95 (5% reduction per step — gradual communication degradation)

Applied to cyber features only. Physical readings remain unaffected. Requires 2-of-3 Layer C conditions to fire.

### Man-in-the-Middle (MITM)

- Physical features: += N(0, 1.0^2) Gaussian noise per timestep

- Cyber latency: += N(0, 0.5^2) noise per timestep

- Cyber pkt_loss: += N(0, 0.02^2) noise per timestep

- Integrity: -= 0.10 per timestep (deterministic decay, floored at 0)

The oscillatory Gaussian noise pattern in physical features (OSCILLATORY temporal signature) is what Layer C-3 detects alongside the integrity decay.

## 4.2 Anomaly Generation — Rapid SCADA Mode

In Rapid SCADA Live workspace, there is NO synthetic attack injection. Anomalies are detected purely by deviation from per-type baselines.

- Rapid SCADA calculates all 670 channel values using built-in formula expressions stored in Cnl.xml (physical) and Cnl_Cyber_Addon.xml (cyber)

- These formulas introduce controlled variation around the baseline values

- When calculated values deviate beyond the normalised threshold (z_j > k_sigma), the deviation scoring pipeline treats them as anomalous

- No physical injection — the detection responds to whatever the SCADA formulas produce

- Detection pipeline in SCADA mode is identical to Experiment mode: PowerShell bridge → FastAPI → normalisation → LSTM → Layer C → Tier-A → dashboard

# 5. Multi-Layer Detection Pipeline

At every time step, the pipeline receives each agent's feature vector and returns: (a) binary anomaly flag, (b) attack-type label (FDI/DoS/MITM/FAULT/CHAIN), (c) XAI feature attribution.

## 5.1 Layer A — Statistical Deviation Scoring

Physical deviation: dx = sqrt[ (1/nx) x Sum( ((xj - bxj) / tau_xj)^2 ) ]

Cyber deviation:  dy = sqrt[ (1/ny) x Sum( ((yj - byj) / tau_yj)^2 ) ]

Composite score: S_i(t) = w_i x (dx + dy)

Criticality weights: GEN=1.0, PMU=0.8, SUB=0.7, BRK=0.5

An agent is flagged by Layer A when BOTH hold: LSTM probability >= 0.80 AND deviation score >= 3.60 (BALANCED profile). Both conditions must be met — neither alone is sufficient.

## 5.2 Three Detection Sensitivity Profiles

| Parameter | ROBUST | BALANCED (default) | COST |
| --- | --- | --- | --- |
| k_sigma | 3.5 | 4.0 | 5.0 |
| Deviation score threshold | 3.60 | 3.85 | 4.10 |
| LSTM probability threshold | 0.80 | 0.85 | 0.90 |
| Use case | High-security | General operations | Budget-constrained |

## 5.3 Layer B — LSTM Temporal Accumulator

- 2-layer LSTM, 64 hidden units per layer, dropout = 0.20

- Fully connected output → sigmoid → probability in [0, 1]

- Sliding window: 24 consecutive timesteps per training sample

- Stage 1 fusion: fused_prob = 0.58 x grid_branch + 0.42 x network_branch + agreement bonus - disagreement penalty

- Temporal accumulator: flags when LSTM probability >= 0.55 for 5+ consecutive steps within a 6-step window

Class imbalance handling: Focal Loss (alpha=0.65, gamma=2.0) + WeightedRandomSampler (positive scale=1.40, oversampling=1.35). Temperature calibration: grid search over T in [0.80, 2.20] to maximise F1 and minimise Brier score.

## 5.4 Stage 2 Ensemble Fusion

hybrid_conf = 0.48 x dev_conf + 0.52 x fused_LSTM_prob + suspicion_credit

suspicion_credit accumulates when borderline readings persist and decays on consecutive normal readings.

## 5.5 Layer C — Three Attack-Specific Detectors

### C-1: CUSUM for FDI

Two-sided CUSUM. Sensitivity k=0.50, alarm threshold h=4.00, window 8 steps. Fires when cumulative physical-feature residuals breach h=4.0 — calibrated to catch x_attack(t) = x(t)+2.0+0.05t after 8 steps.

### C-2: DoS Rule Detector

2-of-3 rule: fires when at least 2 of these 3 conditions are simultaneously satisfied: (1) latency >= 3x baseline, (2) packet loss >= 0.15, (3) comm frequency drop >= 40%.

### C-3: MITM Integrity Detector

AND-gate: (1) integrity drop >= 35% from baseline AND (2) normalised measurement jump >= 2.5 sigma over 8-step history. Distinguishes MITM from operational transients.

## 5.6 Tier-A False Positive Suppression

ALL 5 conditions must hold simultaneously to suppress an alert:

- Score ratio < 3.50 (deviation is marginal)

- LSTM anomaly probability < 0.60

- Network intrusion probability < 0.55

- Hybrid confidence < 1.05

- No Layer C signature present (no FDI/DoS/MITM/FAULT indicator)

If even ONE condition fails, the alert is retained. This gate only suppresses very clear non-events.

## 5.7 Temporal Signature and 5-Class Attack Typing

| Signature | Pattern | Primary Attack |
| --- | --- | --- |
| STEP | Abrupt sustained shift | FDI with constant bias |
| RAMP | Linearly increasing deviation | FDI with drift, or gradual fault |
| OSCILLATORY | Alternating +/- deviations | MITM noise injection |
| STABLE | No significant pattern | Normal operation or very slow fault |

| Attack Type | Detection Criteria |
| --- | --- |
| FDI | Physical deviation STEP/RAMP + CUSUM fires + cyber normal |
| DoS | Network rule fires + 2-of-3 conditions + physical normal |
| MITM | Integrity-jump fires + OSCILLATORY signature + both domains elevated |
| FAULT | Physical spike + no cyber degradation + STEP + no network indicator |
| CHAIN | Multiple Layer C detectors fire + both physical and cyber elevated |

# 6. Training Datasets

Two benchmark datasets pre-train the LSTM branches before live training. They do NOT appear in the live telemetry pipeline — they only provide the initial weight prior.

## 6.1 UCI Electrical Grid Stability Dataset (Physical LSTM Branch)

- UCI Repository Dataset #471

- 10,000 samples, 12 features + 1 binary label

- Binary label: stable (0) / unstable (1) — 70% stable / 30% unstable

- Teaches the physical LSTM what power-flow instability looks like before live training

### UCI Feature Mapping — Table 4.1

| UCI Feature Group | UCI Columns | Maps to Simulation Feature | Rationale |
| --- | --- | --- | --- |
| Response times | tau1, tau2, tau3, tau4 | response_time | Producer reaction time maps directly to control latency |
| Power coefficients | p1, p2, p3, p4 | voltage_deviation / power_deviation | Power balance imbalance reflects voltage and power deviations |
| Price elasticity | g1, g2, g3, g4 | frequency_deviation / current_deviation | Elasticity relates to load-frequency and current dynamics |

The mapping is not direct injection. UCI features are averaged into groups and used as pre-training data so the physical LSTM learns power-flow instability patterns from a labelled dataset before it encounters live simulation or SCADA data.

## 6.2 UNSW-NB15 Network Intrusion Dataset (Cyber LSTM Branch)

- UNSW Canberra Cyber Lab — real captured network traffic

- 257,675 packets: 82K train / 175K test

- Binary label: normal (0) / attack (1)

- Teaches the cyber LSTM what network-level attack signatures look like

### UNSW-NB15 Feature Engineering — Table 4.2

| Composite Feature | Formula | Source Columns | Maps to Simulation Feature |
| --- | --- | --- | --- |
| latency_like | ln(1 + sum) | dur + tcprtt + synack + ackdat + sinpkt + dinpkt + sjit + djit | latency |
| loss_like | ln(1 + sum) | sloss + dloss + pkt_correction | pkt_loss |
| integrity_like | ln(1 + sum) | ¦sttl-dttl¦ + ct_state_ttl + ¦sbytes-dbytes¦ / max(sbytes+dbytes, 1) | integrity |
| freq_like | ln(1 + sum) | rate + sload + dload + spkts + dpkts | comm_freq |

Processing pipeline: (1) Sum all source columns, (2) Apply ln(1+x) transform — handles zeros and compresses skew, (3) Z-score normalise — brings all features to unit scale. All four composite features then map one-to-one to the 4 simulation cyber features.

# 7. Hybrid Audit Scheduler

Two complementary methods combined. Q-learning handles which escalation level. Gradient descent handles how much audit resource to allocate. Neither method alone covers both dimensions.

## 7.1 Q-Learning Component

State: s = (b_risk, b_prob, c_cluster, b_cap) — 4-dimensional, 300 total states (5x5x3x4)

| Dimension | Values | Edge Thresholds |
| --- | --- | --- |
| b_risk (risk score bucket) | 5 levels | 0 / 0.5 / 1 / 2 / 5 / 10 |
| b_prob (LSTM probability bucket) | 5 levels | 0 / 0.2 / 0.4 / 0.6 / 0.8 / 1 |
| c_cluster (K-Means cluster) | 3 clusters | k=3 on agent telemetry history |
| b_cap (audit capacity bucket) | 4 levels | 0 / 0.5 / 0.8 / 1 / 2 |

Bellman update: Q(s,a) = Q(s,a) + alpha x [r + gamma x max Q(s',a') - Q(s,a)]

alpha=0.10, gamma=0.90, epsilon-greedy starting at 1.0, decaying x0.995 to min 0.05

Three actions: DEC (decrease audit frequency) | HOLD | INC (increase)

Experience replay: buffer capacity=2000, mini-batch=32. Three reward modes: Expected, Exponential Utility (beta=-0.05), CVaR (alpha=10%).

## 7.2 Gradient Descent Cost Optimisation

Cost function: C_i = C_a x f_i + C_f x (R_i / f_i)

Gradient: dC_i/df_i = C_a - C_f x (R_i / f_i^2)

Update: f_i = f_i - eta x (dC_i/df_i)  with  eta=0.01

Optimal frequency: f_i* = sqrt(C_f x R_i / C_a)

Result: cost efficiency improves from 42.5% (Q-only, base paper) to 57.28% (hybrid).

## 7.3 Escalation Actions — Five Decision Levels

| Level | Action | Risk Score Range | Response |
| --- | --- | --- | --- |
| 1 | NO ANOMALY | Score < 1.0 | No action; normal monitoring cycle continues |
| 2 | LOG MONITOR | Score 1.0-2.0 | Event logged to audit_chain.db; monitoring frequency increased |
| 3 | INCREASE AUDIT | Score 2.0-3.5 | Audit frequency escalated for agent AND its immediate neighbours |
| 4 | ISOLATE NOTIFY | Score 3.5-5.0 | Agent flagged for isolation; operator alert triggered; XAI delivered |
| 5 | EMERGENCY SHUTDOWN | Score > 5.0 | Emergency protocol triggered; all stakeholders notified; trail sealed |

# 8. Explainability Module (XAI)

Per-feature contribution: c_j = ((x_j - b_j) / tau_j)^2

Relative importance: relative_j = c_j / Sum_k(c_k)

Physical and cyber domains ranked independently. Top-k features shown per domain.

No additional inference passes needed — reuses deviation values already computed. Physical features and cyber features have separate leaderboards per alert.

Example output: "Agent GEN-07 flagged: voltage deviation 41.2%, latency anomaly 23.1%, packet loss 18.8%; likely FDI attack."

# 9. System Testing — 12 Integration Test Cases

All 12 test cases passed. Tests cover detection, scheduling, SCADA integration, and XAI. Test environment: Experiment Running workspace, seed 43, BALANCED profile, 100 agents.

| Test Case | Description | Expected / Result |
| --- | --- | --- |
| TC-01 | Single FDI on one GEN agent | FDI detected; CUSUM fired within 8 steps — PASS |
| TC-02 | DoS attack on one SUB agent | DoS detected; 2-of-3 corroboration rule satisfied — PASS |
| TC-03 | MITM attack on one PMU agent | MITM detected; integrity drop + oscillatory signature — PASS |
| TC-04 | Coordinated FDI across 5 GEN agents | CHAIN attack classified; all 5 flagged — PASS |
| TC-05 | Normal operation 30-minute window | Zero false positives over full 30-minute run — PASS |
| TC-06 | Borderline LSTM spike (ROBUST profile) | Layer A catches when deviation also elevated — PASS |
| TC-07 | Slow FDI onset (drift 0.02/step) | Layer B temporal accumulator flags after 5 steps — PASS |
| TC-08 | Simultaneous FDI + DoS injection | CHAIN classification; both Layer C detectors fire — PASS |
| TC-09 | SCADA polling continuity (N=100) | All 100 agents polled and processed every 5s cycle — PASS |
| TC-10 | XAI feature ranking correctness | Injected feature identified as rank-1 driver — PASS |
| TC-11 | Q-learning escalation decision | DEC/HOLD/INC action matches expected risk bucket — PASS |
| TC-12 | End-to-end latency (N=100) | P95 latency = 66 ms, within 5s SCADA interval — PASS |

# 10. Experimental Results

## 10.1 Primary System Metrics (N=100, 24-Hour Evaluation, Five-Seed Mean, Audit Protection Active)

| Metric | Value |
| --- | --- |
| Detection Accuracy | 93.85% |
| False Positive Rate | 6.18% |
| Recall | 98.24% |
| Risk Mitigation | 93.66% |
| Cost Efficiency | 57.28% |
| Audit Coverage | 100.00% |
| Attack Rate Reduction | 76.13% |
| Precision (24h) | 9.77% |
| F1 Score (24h) | 17.76% |
| Per-step F1 (5-method study) | 82.25% |
| Cross-Layer Stability Index | 90.62% |
| Average Pipeline Latency | 55.04 ms |
| P95 Latency | 66.02 ms |

Note on precision and F1: The low 24h-cycle precision (9.77%) and F1 (17.76%) reflect the audit and mitigation pipeline removing most attack events before they accumulate, which leaves a small positive class against a large normal class. A modest count of residual flags then drives precision down even though operational accuracy stays high. The per-step F1 of 82.25% from the five-method comparative study is the accurate measure of per-decision quality.

## 10.2 Multi-Seed Reproducibility — Table 7.6

Five independent evaluation runs, N=100, where each seed places attacks on a different set of agents with a different noise pattern. The reported headline is the five-seed mean, which shows the results are stable across seeds rather than resting on a single run.

| Seed | Detection Accuracy | False Positive Rate | Recall |
| --- | --- | --- | --- |
| 42 | 93.75% | 6.28% | 98.53% |
| 43 | 94.34% | 5.69% | 99.01% |
| 44 | 93.83% | 6.21% | 100.00% |
| 45 | 93.42% | 6.60% | 97.22% |
| 46 | 93.91% | 6.10% | 96.43% |
| Mean | 93.85% | 6.18% | 98.24% |
| Std Dev | 0.33% | 0.33% | 1.42% |

## 10.3 Five-Method Comparative Study

| Method | Accuracy | FPR | Recall | Precision | F1 |
| --- | --- | --- | --- | --- | --- |
| Deviation-Only (base paper) | 81.40% | 9.06% | 36.89% | 46.58% | 41.18% |
| LSTM-Only | 86.35% | 0.00% | 22.67% | 100.00% | 36.96% |
| Isolation Forest | 68.85% | 27.95% | 53.92% | 29.25% | 37.92% |
| One-Class SVM | 46.35% | 59.99% | 75.93% | 21.33% | 33.31% |
| Our System (Multi-Layer) | 94.23% | 1.81% | 75.76% | 89.95% | 82.25% |

## 10.4 Scalability Analysis

| Agents | Accuracy | FPR | Risk Mit. | Cost Eff. | Coverage | Avg Lat. | P95 Lat. |
| --- | --- | --- | --- | --- | --- | --- | --- |
| N=100 | 93.85% | 6.18% | 93.66% | 57.28% | 100% | 55 ms | 66 ms |
| N=200 | 93.69% | 6.33% | 93.76% | 65.76% | 100% | 103 ms | 124 ms |
| N=500 | 93.70% | 6.33% | 94.05% | 89.95% | 91.44% | 255 ms | 303 ms |

Detection scales cleanly: accuracy stays within a fifth of a percentage point of 93.7% and false positive rate near 6.3% at all three sizes, while risk mitigation holds near 94%. Audit coverage stays at 100% to N=200 and falls to 91.44% at N=500, and cost efficiency rises from 57.28% to 89.95% as the fixed budget ratio conserves a larger share of audits. The resources that need attention for larger deployments are the audit budget and inference batching, not the per-step detection itself.

# 11. Operator Dashboard

Built with Next.js — two workspaces, eight views. All views update on every SCADA poll cycle (5-second interval at N=100).

- Operations Overview: system-wide summary — headline metrics, top-10 risk-ranked agents, anomaly trend

- Agent Monitoring: per-agent drill-down with live telemetry traces, 4-colour status (green/yellow/orange/red)

- Risk Analytics: risk score histogram, 24h trend, heatmap by agent type and hour

- Audit Trail: hash-chained log of all audit decisions, searchable by agent/severity/time

- Threat Events: live timeline of detected attacks with type labels, confidence, timestamps

- Decision Explainability: feature attribution bar chart, text summary, Q-table action taken

- Asset Topology: 10x10 grid of agent icons colour-coded by anomaly status with dependency links

- Experiment Control / System Health: configure evaluation runs; SCADA bridge connectivity status

# 12. Key Formulas Reference

| Formula | Equation | Where Used |
| --- | --- | --- |
| Normalisation | z_j = (x_j - b_j) / tau_j | Every detection step, all features |
| Physical deviation | dx = sqrt[(1/nx) x Sum(z_j^2)] | Layer A deviation scoring |
| Composite score | S_i(t) = w_i x (dx + dy) | Layer A, Tier-A gate |
| Stage 1 LSTM fusion | fused = 0.58 x grid + 0.42 x net + bonuses - penalties | Layer B output |
| Stage 2 ensemble | hybrid = 0.48 x dev_conf + 0.52 x fused_LSTM + suspicion_credit | Detection output |
| XAI contribution | c_j = ((x_j - b_j) / tau_j)^2 | Explainability module |
| Bellman update | Q(s,a) = Q(s,a) + 0.10 x [r + 0.90 x max Q(s',a') - Q(s,a)] | Q-learning scheduler |
| Gradient cost | C_i = C_a x f_i + C_f x (R_i / f_i) | Cost optimisation |
| Optimal frequency | f_i* = sqrt(C_f x R_i / C_a) | Gradient descent target |
| FDI injection | x_attack(t) = x(t) + 2.0 + 0.05 x t | Experiment mode simulation |
| Focal Loss | L = -alpha x (1 - pt)^gamma x log(pt) | LSTM training (alpha=0.65, gamma=2.0) |
| Baseline EMA | b_j(t+1) = 0.90 x b_j(t) + 0.10 x x_j(t) if no anomaly | Baseline update |

# 13. Limitations and Future Work

## 13.1 Acknowledged Limitations

| Limitation | Detail |
| --- | --- |
| Calculated SCADA channels | Telemetry generated within Rapid SCADA — no physical RTUs, IEDs, or PMUs connected |
| Operational prototype | Not a field-deployed utility system — validated under simulated telemetry |
| Fixed baselines | Hand-engineered per-type baselines — not auto-learned from operational data |
| Audit budget at large scale | Detection stays stable to N=500 (FPR near 6.3%), but audit coverage falls to 91.44% at N=500 as the fixed budget ratio is spread across more agents |
| 24h F1 of 17.76% | Reflects the audit and mitigation pipeline clearing most attacks before they accumulate, leaving a small positive class — not model weakness |
| Attack typology | 5 classes modelled — replay attacks and ramp-metering attacks not yet covered |

## 13.2 Future Work (7 Directions)

- 1. Replace calculated SCADA channels with physical RTU / IED / PMU telemetry via Modbus or OPC-UA

- 2. Adaptive baseline models that adjust to seasonal load variation, equipment ageing, and topology changes, reducing reliance on static reference profiles

- 3. Per-scale adaptive threshold calibration for deployments beyond 200 agents

- 4. Extended attack typology: replay attacks, load redistribution, ramp-metering attacks

- 5. IDS / SIEM integration for coordinated enterprise-level threat response

- 6. Adversarial robustness testing against adaptive attackers who probe system thresholds

- 7. Formal verification of safety-critical escalation decisions using model checking

# 14. Glossary of Key Terms

| Term | Plain-Language Definition |
| --- | --- |
| Smart Grid | A power grid that uses computers, sensors, and two-way communication alongside physical wires and transformers |
| SCADA | Supervisory Control and Data Acquisition — software operators use to monitor and control physical equipment remotely |
| FDI | False Data Injection — attacker sends fake sensor readings so operators see normal numbers while the grid is actually misbehaving |
| DoS | Denial of Service — attacker floods the communication channel so real control signals cannot get through |
| MITM | Man-in-the-Middle — attacker sits between two devices, intercepting or modifying messages in transit |
| LSTM | Long Short-Term Memory — a type of neural network that can remember patterns over time, good for detecting slow-onset attacks |
| CUSUM | Cumulative Sum — adds up small deviations over time; catches persistent bias that any single reading would miss |
| Focal Loss | A modified loss function that forces the model to focus on hard, rare examples (attacks) rather than easy normal samples |
| Q-learning | A reinforcement learning algorithm — system learns which actions lead to best outcomes through trial and error |
| Bellman equation | The update rule for Q-learning: new estimate = old estimate + learning_rate x (reward + future value - old estimate) |
| XAI | Explainable AI — provides a ranked list of which features caused each anomaly flag, so operators can verify decisions |
| Rapid SCADA | Open-source SCADA platform (version 6) used as the live supervisory system in this project |
| Baseline | The expected normal value for a feature; used as the reference point for deviation scoring |
| Tier-A Gate | A 5-condition suppression filter that drops very clear false alarms before they reach the operator |
| Temperature Calibration | A post-training step that adjusts LSTM output so 0.80 probability really means 80% empirical attack likelihood |
| Calculated Channel | A SCADA channel that derives its value from a formula expression rather than a physical sensor reading |
| EMA | Exponential Moving Average — a smoothing technique that gives more weight to recent values; used for baseline updates |
| CVaR | Conditional Value at Risk — a risk measure that focuses on the worst-case tail outcomes; used as Q-learning reward mode |
| Attack Rate Reduction | The percentage reduction in active attack count after the system intervenes — 76.13% in the 24h evaluation (five-seed mean) |

--- End of Knowledge Document v2 ---